The GL.iNet GL-SFT1200 (Opal) can absolutely be used standalone in your cruise cabin to create a private local WiFi network with no internet connection required — exactly what you need for your Legion 5 and MacBook Air to communicate directly (via Tailscale or other local sharing).How It Works in Standalone / Offline Mode

- Supported Modes for Private Network:

  - Router Mode (default): Creates its own isolated private network (SSID + password) with NAT/firewall/DHCP. Your two laptops connect to it → they get local IPs (e.g., 192.168.8.x range) → Tailscale detects the local subnet and routes peer-to-peer traffic directly at full WiFi speed (no cloud relay needed).

  - Access Point (AP) Mode: Turns it into a pure wireless access point (broadcasts SSID, no NAT/DHCP). Connect laptops → they can still talk locally (Tailscale works fine here too).

  - Extender / Repeater Mode (with no upstream WiFi selected): Can also be forced into local-only by not joining any external network.

- No WAN / No Internet Needed:

  - Leave the WAN port empty and don't configure any Repeater / Tethering / Cellular uplink.

  - The router still broadcasts its own WiFi SSID and allows devices to connect and communicate locally (LAN-to-LAN traffic).

  - GL.iNet docs and user reports confirm this: You can run it purely for local networking (e.g., offline device sharing, smart home control, or private VPN mesh) without any upstream connection.

  - Cruise users on forums (including GL.iNet and Reddit) often use it this way in cabins for private device-to-device access when ship's WiFi is unreliable or unwanted.

- Setup Steps (Standalone Private Network):

  - Plug in the Opal (USB power from cabin outlet, laptop charger, or power bank).

  - Power on → it auto-creates a default SSID (printed on bottom label, e.g., GL-SFT1200-xxx) with password "goodlife" or similar.

  - Connect both your Legion 5 and MacBook Air to that SSID (change password in admin panel if desired).

  - Access admin interface (http://192.168.8.1 from a connected laptop) → confirm Network Mode is "Router" (default) or switch to "Access Point" if you prefer no NAT.

  - Tailscale should auto-detect the local connection → devices see each other via Tailscale IPs/hostnames → file sharing, remote desktop, ComfyUI sync, etc., all work locally.

  - No ship's WiFi involved — fully isolated.

- Optional Switch to Ship Uplink Later:

  - Go to admin panel → Internet → Repeater → Scan → Join ship's WiFi → authenticate portal once (via one connected laptop).

  - Router shares ship internet to your private SSID (MAC cloning available in advanced settings to count as one device).

  - Toggle back to standalone anytime by disabling the repeater connection.

Yes, the GL.iNet GL-SFT1200 (Opal) can fully work with your iPhone as a hotspot to provide internet to your two portables (Legion 5 and MacBook Air) while still maintaining your private cabin network.It supports two main ways to use your iPhone's Personal Hotspot as the internet source (WAN/uplink), and both keep your private SSID active for the laptops to connect to.1. WiFi Repeater / Hotspot Mode (Wireless Uplink – Recommended for Ease)

- The Opal connects wirelessly to your iPhone's Personal Hotspot (just like any other WiFi network).

- It then shares that internet connection to its own private SSID (which your two laptops connect to).

- Tailscale runs locally over the private network (fast peer-to-peer between laptops) while also allowing internet access via the iPhone hotspot when needed.

- Setup is straightforward:

  - Enable Personal Hotspot on iPhone (Settings > Personal Hotspot > Allow Others to Join).

  - Power on Opal → connect one laptop to Opal's default SSID.

  - Log into Opal admin panel (192.168.8.1) → Internet → Repeater → Scan → Select your iPhone's hotspot SSID → Enter password → Apply.

  - Once connected (green dot shows), Opal now passes iPhone internet to its private SSID.

  - Connect both laptops to Opal's private SSID → they get internet from iPhone + local Tailscale routing.

- Pros: No cables, easy to toggle (disable repeater to go offline/private-only).

- Cons: Slightly slower than wired due to double WiFi hop (iPhone → Opal → laptops), but fine for browsing/email/light use on cruise.

2. USB Tethering (Wired Uplink – More Reliable/Stable)

- Connect iPhone to Opal's USB 2.0 port with a Lightning/USB-C cable.

- Enable Personal Hotspot on iPhone → iPhone shares internet via USB (tethering).

- Opal detects it automatically (or click Connect in admin panel → Internet → Tethering).

- Internet is shared to Opal's private SSID → your laptops connect wirelessly to Opal.

- Pros: More stable than WiFi repeater (no wireless interference), charges iPhone at the same time (if cable supports power delivery), lower latency.

- Cons: Requires cable (keep a short one handy), USB 2.0 speed limit (but plenty for cruise internet).

Additional Cruise Tips for This Setup

- Data Limits / Battery: Use iPhone hotspot sparingly (cruise data is expensive/slow) → turn off when not needed. Opal runs low-power so your iPhone battery drain is minimal.

- MAC Cloning (if Cruise Counts Devices): If the ship limits devices per package, clone your iPhone's MAC address in Opal admin (Advanced → MAC Clone) so the hotspot counts as only one authorized device.

- Captive Portal: If ship WiFi requires login (rare for hotspot, but possible), authenticate once via a laptop connected to Opal.

- Standalone Fallback: At any time, disable tethering/repeater in admin panel → Opal reverts to pure private local network (no internet, laptops talk via Tailscale locally).

- Test Before Cruise: At home, enable iPhone hotspot → connect Opal via WiFi repeater or USB → connect both laptops → confirm internet works + Tailscale pings locally.

Bottom Line: Yes — the GL-SFT1200 Opal handles this perfectly (both standalone private mode and iPhone hotspot uplink via WiFi repeater or USB tethering). It's a popular cruise choice for exactly this scenario. At €38.99, it's low-risk and flexible. Grab it with tomorrow's delivery — you'll have full control over private/offline vs. shared internet in the cabin. Safe travels! If you need exact admin screenshots or MAC clone steps once it's in hand, let me know.