GL.iNet Opal (GL-SFT1200) + Surfshark

Setup Guide: Cruise Travel & Home Use

# Part 1: Cruise Ship Setup

The GL.iNet Opal acts as a personal VPN-secured WiFi hotspot aboard the ship. Your devices connect to the Opal, which connects to the ship's WiFi, with Surfshark tunneling your traffic for privacy.

## What You Need

- GL.iNet Opal router (fully charged or plugged in via USB-C)

- Surfshark account credentials (email + password)

- A laptop or phone to access the Opal admin panel

- Short ethernet cable (optional but useful for a wired fallback)

## Step 1 — First-Time Opal Configuration (do this at home before you leave)

Configure the Opal before the cruise so you're not troubleshooting on the ship's slow WiFi.

- Power on the Opal and connect your laptop to its WiFi network. Default SSID and password are printed on the bottom of the unit.

- Open a browser and go to http://192.168.8.1 — this is the Opal admin panel.

- Set a new admin password when prompted. Write it down somewhere safe.

- Go to VPN > WireGuard Client in the left menu.

- Click Add Manually or use the Surfshark configuration option if available.

- In a separate browser tab, log into your Surfshark account at my.surfshark.com > VPN > Manual Setup > Router > WireGuard. Generate and download a config file for your preferred server location.

- Back in the Opal admin panel, upload or paste the WireGuard config file.

- Enable the VPN connection and confirm the status shows Connected.

- Rename your Opal's WiFi network (SSID) to something personal so you recognize it on the ship.

Tip: Test this at home by browsing to whatismyip.com while connected through the Opal. The IP shown should be a Surfshark server, not your home IP.

## Step 2 — Connecting on the Ship

- Plug in the Opal via USB-C to a cabin USB port or a portable battery.

- Connect your laptop or phone to the ship's WiFi network directly (not through the Opal yet).

- Open the ship's captive portal login page and authenticate with your cruise WiFi credentials. This step must be done device-to-ship directly — the Opal cannot complete a captive portal on your behalf.

- Once authenticated, disconnect from the ship's WiFi on your device.

- Go into the Opal admin panel (http://192.168.8.1) from a device connected to the Opal.

- Go to Internet > Repeater and scan for the ship's WiFi network.

- Select the ship's network and enter the WiFi password. The Opal will now use the ship's connection as its upstream internet source.

- Verify the VPN is active: VPN > WireGuard Client should show Connected.

- All your devices can now connect to the Opal's WiFi and are automatically routed through Surfshark.

Note: Some cruise lines re-authenticate every few hours. If internet stops working, reconnect one device directly to the ship WiFi and re-authenticate through the portal, then reconnect to the Opal.

## Step 3 — If VPN Won't Connect on the Ship

Cruise ship networks sometimes block standard VPN ports. Try these in order:

- In the Opal admin panel go to VPN > WireGuard Client > your config > Edit.

- Change the port to 443 (HTTPS port — almost never blocked) and save.

- If still blocked, try port 80. If neither works, the ship may be blocking WireGuard entirely — in that case, connect without VPN for the trip and use Surfshark's app directly on each device instead.

Reminder: Without VPN on a shared ship network, avoid banking or sensitive logins. Use a cellular data connection for those if possible.

# Part 2: Home Setup — Samsung Frame TV

The Samsung Frame (and most smart TVs) cannot run a VPN natively. By connecting the TV through the Opal, all of the TV's traffic is automatically tunneled through Surfshark — enabling geo-unblocking of streaming services without any configuration on the TV itself.

## How to Set It Up

- Plug the Opal into a power source near your TV (USB-C to USB-A adapter or phone charger works).

- Run a short ethernet cable from one of the Opal's LAN ports into the ethernet port on your Samsung Frame. If your Frame is not near an ethernet port, you can also connect the TV to the Opal via its separate WiFi broadcast.

- Connect the Opal's WAN port (or configure it in Repeater mode) to your home router — either via ethernet from your TP-Link, or connect the Opal to your home WiFi network as an upstream source.

- Log into the Opal admin panel (http://192.168.8.1) and confirm the WireGuard VPN shows as Connected.

- On the Samsung Frame, go to Settings > General > Network and connect to the Opal's network (either wired via ethernet, or wireless to the Opal's SSID).

- Test by opening a streaming app on the Frame — it should now appear as if you're browsing from the Surfshark server location you configured.

Tip: If you configured a US-based Surfshark server, streaming apps on the Frame will see a US IP. Change the WireGuard config server in the Opal admin panel to any country you want for geo-unblocking purposes.

## Switching Between Cruise and Home Configs

The Opal remembers multiple WireGuard configurations. You can save a cruise-optimized config (closest server to your destination) and a home config separately, then activate whichever you need from the admin panel under VPN > WireGuard Client.

- At home: activate your home/streaming server config, connect via ethernet to TP-Link router.

- On cruise: switch to Repeater mode, activate cruise server config, connect to ship WiFi.

- Switching takes under 2 minutes via the admin panel at http://192.168.8.1.

## Quick Reference — Opal Admin Panel

- Admin panel URL: http://192.168.8.1

- VPN config: VPN > WireGuard Client

- Repeater mode (cruise): Internet > Repeater

- Router mode (home): Internet > Cable or Internet > Wireless

- Status check: Dashboard shows WAN status and VPN status at a glance

GL.iNet Opal supports Surfshark WireGuard natively. For firmware updates, visit gl-inet.com/products/gl-sft1200. Surfshark WireGuard configs can be regenerated anytime at my.surfshark.com.